# DWR- 512 Firmware Release Note

Firmware: v1.01B05  (The first official release)

Hardware: A1

Date: 2011/07/26

Existing Issues:

1. There is no information about WPS connection count down to 120 secs.

2. It can allow user setting WAN IP the same with LAN IP.

3. When setting DNS server will ask must enter secondary DNS server IP address , it should be option .

4. We can enter the same DNS server IP address to Primary and secondary DNS server field .

5. DUT cannot use windows 7 WPS PIN code to configure its wireless settings when DUT is un-configured.

6. [#DBG11050262] E-mail function works improperly.

7. [Virtual Server loopback test] LAN PC2 cannot access FTP server by the WAN IP of the DUT even without using MAC filter function.

8. 802.11N (WPA2-PSK-AES) throughput with DWA-645 is too low. See the attachment.

9. 802.11N throughput with DWA-160 is too low. See the attachment.

10. User doesn't modify anything, but DUT will reboot after clicking Save button. For example: DHCP mode, L2TP mode.

11. DUT uses private IP to transmit TCP packets to public IP. This behavior is improper.